CoAP Protocol Overview

Constrained Application Protocol — ESP8266/IoT Reference

# 1. What is CoAP?

CoAP (Constrained Application Protocol) is a lightweight RESTful protocol designed for constrained devices like microcontrollers. It runs over UDP (not TCP), making it ideal for battery-powered ESP8266 nodes that wake from deep sleep, send one sensor reading, and immediately return to sleep — with no TCP state to maintain between cycles.

CoAP uses the same REST-like verbs as HTTP: GET, POST, PUT, and DELETE. Resources are identified by URIs (e.g., /sensors/temp), and responses follow a request-response model similar to HTTP — but with far less overhead.

# 2. Message Types

## 2.1 CON — Confirmable Messages

Confirmable messages require an acknowledgment (ACK) from the receiver. If no ACK is received within a timeout window, the sender retransmits the message. CON is used when delivery guarantee is critical.

- Analogous to TCP's reliability but at the application layer

- Uses exponential back-off for retransmissions (default: up to 4 retries)

- Best for: critical alerts, actuator commands (e.g., open valve)

- Power cost: higher — device must stay awake awaiting ACK

## 2.2 NON — Non-Confirmable Messages

Non-Confirmable messages are fire-and-forget. The sender transmits the packet and immediately proceeds — no ACK expected, no retransmission if lost.

- Analogous to UDP datagrams — lowest overhead possible

- Best for: periodic sensor telemetry (temperature, humidity) where occasional loss is acceptable

- Power cost: lowest — transmit and sleep immediately

- Typical use in Zelbytes polyhouse: DHT22 readings every 55 s over NON CoAP

## 2.3 OBSERVE — Push Subscription Pattern

The OBSERVE option extends CoAP's GET method to enable a publish-subscribe (push) model without a broker. A client registers its interest in a resource; the server then pushes notifications whenever the resource value changes.

- Client sends: GET /sensors/temp with Observe: 0 (register)

- Server responds immediately with current value, then pushes updates on change

- Client cancels with Observe: 1 or by not acknowledging

- Eliminates the need for an MQTT broker in simple push scenarios

- Best for: real-time dashboards monitoring soil moisture or CO2 levels

# 3. CoAP vs MQTT — Quick Comparison

| Dimension | CoAP | MQTT |
| --- | --- | --- |
| Transport | UDP | TCP |
| Broker needed | No | Yes (e.g., Mosquitto) |
| Model | Request/Response + OBSERVE | Publish/Subscribe |
| Overhead | 4-byte header (minimal) | 2-byte fixed + variable |
| Deep sleep fit | Excellent — no connection state | Poor — TCP reconnect overhead |
| QoS | CON/NON (app-layer) | 0, 1, 2 (broker-managed) |
| Discovery | CoRE Link Format (/.well-known/core) | Topic naming convention |

# 4. CoAP Resource Mapping — Polyhouse Example

CoAP resources are addressed by URI path. A GET to /sensors/temp returns the current DHT22 value as a CoAP response payload.

| Resource URI | Method | Description |
| --- | --- | --- |
| /sensors/temp | GET | Returns current DHT22 temperature (°C) |
| /sensors/humidity | GET | Returns DHT22 relative humidity (%) |
| /sensors/soil | GET | Returns soil moisture ADC reading |
| /actuators/valve | PUT | Sets irrigation valve state (on/off) |
| /sensors/temp | GET + OBSERVE | Push updates on temperature change |

# 5. Deep Sleep Power Budget Link (Day 22)

CoAP's stateless UDP design directly enables the deep sleep pattern covered in Day 22. The wake cycle for a battery ESP8266 node is:

- Wake from deep sleep (~1 ms)

- Connect to Wi-Fi (~800 ms typical)

- Send NON CoAP POST /sensors/temp with DHT22 reading (~50 ms)

- Return to deep sleep — no ACK wait, no TCP teardown

Total active time: ~900 ms per 55-second cycle. Compare to MQTT which requires TCP connect + MQTT CONNECT + PUBLISH + DISCONNECT — adding ~200-400 ms and significant power overhead per cycle.

See Day 22 Capstone Integration Plan for full power budget calculations and deep sleep timer configuration on ESP8266 using ESP-IDF esp_deep_sleep_start().