Polyhouse Sensor Protocol Selection

CoAP vs MQTT — Use Case Decision Document

# 1. Decision Framework

Protocol selection for each sensor cluster depends on four factors: power source (battery vs mains), required delivery guarantee, need for a persistent data stream, and infrastructure availability (broker or no broker).

| Factor | Choose CoAP | Choose MQTT | Either OK | Notes |
| --- | --- | --- | --- | --- |
| Power source | Battery | Mains | — | CoAP avoids TCP state overhead |
| Data frequency | Periodic bursts | Continuous stream | — | MQTT keeps connection alive |
| Broker available | No | Yes | — | CoAP is brokerless |
| Fan-out subscribers | 1-2 (OBSERVE) | Many | — | MQTT pub/sub scales better |
| Delivery critical | CON mode | QoS 1 or 2 | Both work | CON = app-layer ACK |

# 2. Sensor Cluster Protocol Assignments

## 2.1 Remote Battery Clusters — CoAP (NON)

These nodes are powered by 18650 Li-ion cells and must maximise deep sleep duration. They wake every 55 seconds, sample sensors, transmit one CoAP NON POST, and return to sleep.

| Sensor | Protocol | Message Type | Resource URI |
| --- | --- | --- | --- |
| DHT22 — Temperature | CoAP | NON POST | /sensors/temp |
| DHT22 — Humidity | CoAP | NON POST | /sensors/humidity |
| Soil Moisture (ADC) | CoAP | NON POST | /sensors/soil |
| Light (LDR) | CoAP | NON POST | /sensors/light |

## 2.2 Mains-Powered Aux Nodes — MQTT

Mains-powered nodes run continuously and benefit from MQTT's persistent TCP connection and broker-managed fan-out. Dashboard, logging service, and alert system all subscribe to the same topic.

| Sensor / Actuator | Protocol | QoS Level | Topic |
| --- | --- | --- | --- |
| CO2 Sensor (MH-Z19) | MQTT | QoS 1 | polyhouse/co2 |
| Irrigation Valve | MQTT | QoS 2 | polyhouse/valve/cmd |
| Gateway Aggregator | MQTT | QoS 1 | polyhouse/gateway/status |
| Fan / Ventilation | MQTT | QoS 2 | polyhouse/fan/cmd |

## 2.3 Hybrid: CoAP OBSERVE for Real-Time Monitoring

For the central monitoring dashboard that needs live updates from a specific sensor without an MQTT broker, CoAP OBSERVE provides a lightweight push mechanism:

- Dashboard client sends GET /sensors/temp with Observe: 0 to the ESP8266 server

- Node pushes temperature updates whenever value changes by more than 0.5°C

- No broker infrastructure required — direct device-to-dashboard link

- Suitable for on-site technician tablet during manual inspection rounds

# 3. Summary Decision Tree

When assigning a new polyhouse sensor to a protocol, use this logic:

- Is the node battery-powered AND sends periodic readings? → CoAP NON

- Is delivery of a command (e.g., open valve) critical? → CoAP CON or MQTT QoS 2

- Are there multiple subscribers (dashboard + logger + alert)? → MQTT

- Is there no broker in the field cluster? → CoAP

- Is the node mains-powered and always-on? → MQTT (keep connection alive)