Power Budget Estimate

MQTT vs CoAP — 55-Second Wake Cycle (ESP8266)

# 1. Assumptions

All figures are estimates for ESP8266 (ESP-12E module) with DHT22 sensor, 3.3 V supply, 55-second deep sleep cycle, Wi-Fi connecting to a 2.4 GHz access point. Current values are typical from Espressif datasheet and community measurements.

| Parameter | Value | Source |
| --- | --- | --- |
| Deep sleep current | 20 µA | Espressif datasheet |
| Wi-Fi active current | ~80 mA (avg during TX) | Espressif AN |
| DHT22 read time | ~18 ms | DHT22 datasheet |
| Boot time | ~300 ms | ESP8266 SDK |
| Wi-Fi connect time | ~800 ms (typical) | Community measurement |
| Sleep cycle period | 55 seconds | Task specification |
| Battery capacity | 2000 mAh (18650) | Typical Li-ion cell |

# 2. Wake Cycle Timing Breakdown

| Phase | Current | CoAP Duration | MQTT Duration | Notes |
| --- | --- | --- | --- | --- |
| Boot + GPIO init | 80 mA | 300 ms | 300 ms | Same for both |
| Wi-Fi connect | 80 mA | 800 ms | 800 ms | Same for both |
| DHT22 sample | 15 mA | 18 ms | 18 ms | Same for both |
| TCP connect | 80 mA | N/A | ~150 ms | CoAP skips this |
| MQTT CONNECT packet | 80 mA | N/A | ~50 ms | CoAP skips this |
| Protocol data TX | 80 mA | ~50 ms (UDP NON) | ~50 ms (PUBLISH) | Similar |
| MQTT DISCONNECT | 80 mA | N/A | ~30 ms | CoAP skips this |
| TCP teardown (FIN) | 80 mA | N/A | ~100 ms | CoAP skips this |
| TOTAL ACTIVE TIME | 80 mA | ~1168 ms | ~1498 ms | CoAP saves ~330 ms |

# 3. Energy Per Cycle

| Metric | CoAP (NON/UDP) | MQTT (QoS 0/TCP) |
| --- | --- | --- |
| Active time per cycle | ~1168 ms | ~1498 ms |
| Active energy (80 mA) | 25.8 µAh | 33.3 µAh |
| Sleep time per cycle | 53.83 s | 53.50 s |
| Sleep energy (20 µA) | 299.1 µAh | 297.2 µAh |
| TOTAL per cycle | 324.9 µAh | 330.5 µAh |
| Cycles per hour | ~65.5 | ~65.5 |
| Avg current draw | 5.87 mA | 5.97 mA |
| Battery life (2000 mAh) | ~341 hours (~14.2 days) | ~335 hours (~13.9 days) |
| CoAP advantage | +6 hours battery life | — |

# 4. Interpretation

While the per-cycle energy difference between CoAP and MQTT appears small (~5.6 µAh), the cumulative effect over 14+ days of operation results in approximately 6 additional hours of battery life per charge cycle using CoAP.

More significantly, CoAP's advantage grows when Wi-Fi reconnection is slow (rural AP, congested network) or when the MQTT broker is temporarily unavailable and the node must retry — MQTT retries require full TCP reconnection, whereas CoAP NON simply drops the packet and sleeps.

For Zelbytes polyhouse aux sensor clusters in remote greenhouse sections without mains power, CoAP NON is the recommended protocol. MQTT should be reserved for always-on mains-powered nodes where the persistent connection and broker fan-out provide tangible value.

# 5. Day 22 Deep Sleep Capstone Integration

This power budget feeds directly into the Day 22 Capstone Integration Plan, where ESP-IDF's esp_deep_sleep_start() is configured with a 55-second timer. The CoAP NON transmission pattern — transmit and immediately enter deep sleep without waiting for ACK — is the key enabler of this power profile.

Reference: Day 22 Capstone Integration Plan → Section 3: Deep Sleep Timer Configuration and Section 4: CoAP Transmission Window.