POLYHOUSE IOT NODE

Field Deployment Package

Date: _______________   Node ID: _______________   Technician: _______________

# Document 1 — Field Deployment Checklist

Complete all items before marking the deployment done. Mark each item PASS / FAIL / N/A. A FAIL must be resolved or documented before going live.

| # | Category | Check Item | PASS / FAIL / N/A |
| --- | --- | --- | --- |
| 1 | Pre-flight | Verify firmware version matches release notes | ☐ PASS ☐ FAIL ☐ N/A |
| 2 | Pre-flight | Confirm OTA partition scheme is dual-bank | ☐ PASS ☐ FAIL ☐ N/A |
| 3 | Pre-flight | Check MQTT broker IP/hostname in NVS config | ☐ PASS ☐ FAIL ☐ N/A |
| 4 | Pre-flight | Confirm MQTT ACL allows node topic publish/subscribe | ☐ PASS ☐ FAIL ☐ N/A |
| 5 | Pre-flight | Test LWT (Last Will Testament) topic shows 'offline' on disconnect | ☐ PASS ☐ FAIL ☐ N/A |
| 6 | Pre-flight | Verify NTP sync working (correct timestamp in logs) | ☐ PASS ☐ FAIL ☐ N/A |
| 7 | WiFi | Perform RSSI survey at install height (target > -70 dBm) | ☐ PASS ☐ FAIL ☐ N/A |
| 8 | WiFi | Confirm SSID and password stored in NVS (not hardcoded) | ☐ PASS ☐ FAIL ☐ N/A |
| 9 | WiFi | Test reconnect after simulated AP reboot | ☐ PASS ☐ FAIL ☐ N/A |
| 10 | WiFi | Confirm IP plan: static IP or DHCP reservation documented | ☐ PASS ☐ FAIL ☐ N/A |
| 11 | Hardware | Enclosure IP rating verified (IP65 minimum for polyhouse) | ☐ PASS ☐ FAIL ☐ N/A |
| 12 | Hardware | Cable glands installed and tightened on all entry points | ☐ PASS ☐ FAIL ☐ N/A |
| 13 | Hardware | Cable strain relief applied to all internal wiring | ☐ PASS ☐ FAIL ☐ N/A |
| 14 | Hardware | DHT22 sensor positioned per placement plan (shade, airflow) | ☐ PASS ☐ FAIL ☐ N/A |
| 15 | Hardware | Sensor not touching metal frame (thermal isolation confirmed) | ☐ PASS ☐ FAIL ☐ N/A |
| 16 | Hardware | Lightning/ESD grounding wire connected to metal scaffolding | ☐ PASS ☐ FAIL ☐ N/A |
| 17 | Hardware | USB power bank capacity confirmed (≥10,000 mAh) | ☐ PASS ☐ FAIL ☐ N/A |
| 18 | Hardware | Power cable routed away from mist nozzles | ☐ PASS ☐ FAIL ☐ N/A |
| 19 | Testing | Node publishes sensor data within 60s of power-on | ☐ PASS ☐ FAIL ☐ N/A |
| 20 | Testing | MQTT dashboard shows correct temp/humidity readings | ☐ PASS ☐ FAIL ☐ N/A |
| 21 | Testing | 24h soak test completed with no dropouts | ☐ PASS ☐ FAIL ☐ N/A |
| 22 | Testing | OTA update successfully applied in bench test | ☐ PASS ☐ FAIL ☐ N/A |
| 23 | Documentation | Enclosure photo taken with labeled wiring | ☐ PASS ☐ FAIL ☐ N/A |
| 24 | Documentation | Rollback runbook reviewed and accessible on-site | ☐ PASS ☐ FAIL ☐ N/A |
| 25 | Documentation | Maintenance schedule posted inside enclosure lid | ☐ PASS ☐ FAIL ☐ N/A |

Checked by: _____________________     Date: _________________     Sign: _________________

# Document 2 — WiFi RSSI Site Survey

Record RSSI at each location using a WiFi analyzer app (e.g. WiFi Analyzer on Android). Measure at actual install height. Minimum acceptable: -70 dBm. Below -80 dBm = relocate AP or add extender.

## Survey Details

| AP/Router Model |  |
| --- | --- |
| AP Location |  |
| WiFi Channel |  |
| Survey Tool Used |  |
| Survey Date/Time |  |

## RSSI Readings by Location

| # | Location Description | RSSI (dBm) | Signal Quality | Notes / Obstructions |
| --- | --- | --- | --- | --- |
| 1 | Entrance (ground level) | _______ | ☐ Good ☐ Fair ☐ Poor |  |
| 2 | Row 1 - North end (1.5m height) | _______ | ☐ Good ☐ Fair ☐ Poor |  |
| 3 | Row 1 - South end (1.5m height) | _______ | ☐ Good ☐ Fair ☐ Poor |  |
| 4 | Row 2 - Centre (1.5m height) | _______ | ☐ Good ☐ Fair ☐ Poor |  |
| 5 | Row 3 - North end (1.5m height) | _______ | ☐ Good ☐ Fair ☐ Poor |  |
| 6 | Row 3 - South end (1.5m height) | _______ | ☐ Good ☐ Fair ☐ Poor |  |
| 7 | Install point - Scaffolding level (3m) | _______ | ☐ Good ☐ Fair ☐ Poor |  |
| 8 | Install point - Top scaffold (4m) | _______ | ☐ Good ☐ Fair ☐ Poor |  |
| 9 | Far corner from AP | _______ | ☐ Good ☐ Fair ☐ Poor |  |
| 10 | Near misting equipment | _______ | ☐ Good ☐ Fair ☐ Poor |  |

## Signal Quality Reference

| RSSI Range | Quality | Action |
| --- | --- | --- |
| -30 to -60 dBm | Excellent | Ideal. No action needed. |
| -60 to -70 dBm | Good | Acceptable for deployment. |
| -70 to -80 dBm | Fair | Monitor. May cause intermittent drops. |
| Below -80 dBm | Poor | Relocate AP or add WiFi extender before deploying. |

# Document 3 — 24-Hour Soak Test Log

Mount the DHT22 node in its enclosure, power via USB power bank, and record readings every hour for 24 hours (or 4 hours for lab compressed test). This validates stability, sensor accuracy, and MQTT connectivity over time.

| Node ID / Hostname |  |
| --- | --- |
| Firmware Version |  |
| Power Source | USB Power Bank — Model: _______________ Capacity: _______mAh |
| Test Start (Date/Time) |  |
| Test End (Date/Time) |  |
| MQTT Broker |  |

## Hourly Log

| Time | Temp (°C) | Humidity (%) | RSSI (dBm) | MQTT Status | Observations |
| --- | --- | --- | --- | --- | --- |
| T+0h |  |  |  | ☐ Online ☐ Offline |  |
| T+1h |  |  |  | ☐ Online ☐ Offline |  |
| T+2h |  |  |  | ☐ Online ☐ Offline |  |
| T+3h |  |  |  | ☐ Online ☐ Offline |  |
| T+4h |  |  |  | ☐ Online ☐ Offline |  |
| T+5h |  |  |  | ☐ Online ☐ Offline |  |
| T+6h |  |  |  | ☐ Online ☐ Offline |  |
| T+7h |  |  |  | ☐ Online ☐ Offline |  |
| T+8h |  |  |  | ☐ Online ☐ Offline |  |
| T+9h |  |  |  | ☐ Online ☐ Offline |  |
| T+10h |  |  |  | ☐ Online ☐ Offline |  |
| T+11h |  |  |  | ☐ Online ☐ Offline |  |
| T+12h |  |  |  | ☐ Online ☐ Offline |  |
| T+13h |  |  |  | ☐ Online ☐ Offline |  |
| T+14h |  |  |  | ☐ Online ☐ Offline |  |
| T+15h |  |  |  | ☐ Online ☐ Offline |  |
| T+16h |  |  |  | ☐ Online ☐ Offline |  |
| T+17h |  |  |  | ☐ Online ☐ Offline |  |
| T+18h |  |  |  | ☐ Online ☐ Offline |  |
| T+19h |  |  |  | ☐ Online ☐ Offline |  |
| T+20h |  |  |  | ☐ Online ☐ Offline |  |
| T+21h |  |  |  | ☐ Online ☐ Offline |  |
| T+22h |  |  |  | ☐ Online ☐ Offline |  |
| T+23h |  |  |  | ☐ Online ☐ Offline |  |
| T+24h |  |  |  | ☐ Online ☐ Offline |  |

## Test Summary

| Total Readings Expected | 25 (T+0h through T+24h) |
| --- | --- |
| Total Readings Received |  |
| Total Dropouts / Offline Events |  |
| Temp Range Observed | Min: ______°C Max: ______°C |
| Humidity Range Observed | Min: ______% Max: ______% |
| Test Result | ☐ PASS ☐ FAIL |

# Document 4 — Rollback Runbook

Use this runbook if a deployed node stops publishing data to MQTT. Three escalating rollback methods are provided. Always attempt Method A first.

## Symptom Identification

| Symptom | Likely Cause |
| --- | --- |
| No MQTT messages for > 5 minutes | WiFi dropout, broker issue, firmware crash |
| LWT 'offline' status stuck | Node crashed or power lost |
| Sensor values frozen / identical | DHT22 read failure, I2C hang |
| Node unreachable via ping | No WiFi connection, IP conflict |
| Node rebooting repeatedly | OTA partition corrupt, watchdog trigger |

## Method A — Power Cycle (First Resort)

Time required: ~2 minutes. No tools needed.

- Disconnect the USB cable from the power bank.

- Wait 30 seconds.

- Reconnect the USB cable.

- Wait 60 seconds for boot + WiFi connect + MQTT reconnect.

- Check MQTT dashboard. If data resumes → RESOLVED. Log incident.

- If no data after 3 minutes → Proceed to Method B.

## Method B — USB Reflash (Firmware Reinstall)

Time required: ~15 minutes. Requires laptop, USB cable, esptool.

- Take node down from scaffolding and bring to laptop.

- Connect node to laptop via USB.

- Run: esptool.py --port COMx erase_flash  (replaces COMx with your port, e.g. /dev/ttyUSB0)

- Flash known-good firmware binary:

esptool.py --port COMx --baud 460800 write_flash 0x0 firmware_v1.0.0.bin

- Re-enter WiFi credentials and MQTT config via serial monitor or provisioning app.

- Bench test: confirm MQTT publishes for 5 minutes.

- Reinstall on scaffolding.

- If still failing → Proceed to Method C.

## Method C — NVS Factory Reset

Time required: ~10 minutes. Use if node boots but cannot connect (corrupt credentials in NVS).

- Connect node to laptop via USB.

- Open serial monitor (115200 baud).

- Send command to erase NVS partition (or hold BOOT button + press EN during boot to enter provisioning mode).

- Re-provision WiFi SSID, password, MQTT broker IP, node ID via provisioning interface.

- Confirm MQTT LWT topic shows 'online'.

- Reinstall on scaffolding and monitor for 30 minutes.

## Method D — Prior OTA Partition Boot

Time required: ~5 minutes. Use if a recent OTA update caused the issue.

- Connect node to laptop via USB serial.

- Run: idf.py monitor (or use esptool to check boot partition).

- Use espefuse or nvs_partition_generator to set boot partition to OTA_0 or OTA_1 (the prior version).

- Alternatively, use esp_ota_ops in firmware to programmatically roll back:

esp_ota_set_boot_partition(esp_ota_get_next_update_partition(NULL));

- Power cycle. Confirm old firmware boots and MQTT resumes.

- Block future OTA updates until root cause of new firmware is fixed.

## Incident Log

| Date/Time | Symptom | Method Used | Outcome | Technician |
| --- | --- | --- | --- | --- |

## Maintenance Schedule

| Frequency | Task | Last Done |
| --- | --- | --- |
| Weekly | Check MQTT dashboard — all nodes online |  |
| Weekly | Check power bank charge level |  |
| Monthly | Wipe enclosure exterior with dry cloth (mist residue) |  |
| Monthly | Inspect cable glands and strain relief for wear |  |
| Monthly | Check ESD grounding wire continuity |  |
| Quarterly | Run OTA update with latest firmware |  |
| Quarterly | Re-run WiFi RSSI survey (seasonal crop changes affect signal) |  |
| Annually | Replace DHT22 sensor (calibration drift over time) |  |
| Annually | Replace USB power bank battery |  |